Effluent Treatment Plants

Technical Data Sheets

An industrial Effluent Treatment Plant should be configured according to the wastewater source.

| Unit process | Main design data |
| --- | --- |
| Coarse screen | 10–25 mm opening |
| Fine screen | 1–6 mm opening |
| Equalization tank | Normally 6–24 hours of average flow |
| pH correction | Typically pH 6.5–8.5 before biological treatment |
| Rapid mixing | Approximately 1–5 minutes |
| Flocculation | Approximately 15–30 minutes |
| Primary clarification | Approximately 1.5–2.5 hours detention time |
| Aerobic biological reactor | DO 2–3 mg/L |
| Secondary clarification | Based on surface overflow rate, solids loading and SVI |
| Tertiary sand filtration | Commonly 5–15 m/h filtration rate, subject to media and water quality |
| Activated carbon | Used for residual organics, colour, taste, odour or trace contaminants |
| Disinfection | Chlorine, UV or ozonation, depending on reuse requirement |
| Sludge handling | Thickening, dewatering and approved final disposal |